Shell-based BIOS Update Operating Guide

1. Open the computer and locate the USB flash drive. See the attached figure for details.（USB flash drives must support the FAT32 file system and have a capacity of at most 32 GB.）

- Right click the USB flash drive and select the formatting option, See the attached figure.

- 3. During the formatting process, change the file system type to FAT 32 (default), name the volume label as WINPE, and then click "Start" to perform the formatting operation, see the attached figure.

4. Please visit https://www.gmktec.com/ to download the latest BIOS file for the X1 Pro; place the file on your system desktop and extract it. Refer to the illustration for the operation:

- 3. Enter under the path of as illustrated in following figure:

6.Once the extraction is complete, open the folder and copy all its files. For detailed instructions, please refer to the accompanying drawings.

7. Open the just-formatted USB flash drive and paste the copied file into its root directory. After pasting, check whether the file size in the USB flash drive is consistent with the decompression file size, see the attached drawings.

8. Turn off the computer to be swiped (ensure that the USB flash drive is inserted), tap the power key to start up, then quickly press the "F7" key to enter the Quick Start menu and select the made USB flash drive to start. See the attached figure for the specific steps.

- After entering the USB startup menu. The page displays as follows:

All the following pages are interfaces that appear during the BIOS flashing process; do not perform any operations. The flashing process typically takes approximately 3–5 minutes. When the first page below appears again in a loop, remove the USB flash drive, turn off the power, and then restart the device.

- matters need attention:

- 1. During the BIOS, the program will run automatically and limit the external device activity (mouse, keyboard, etc.)

- 2, at this time, do not power off, shutdown, restart, etc

- 3. The BIOS file will be automatically turned on after the swipe-in is completed

- If the power failure, shutdown or restart of the machine, the machine may cause the equipment to be used normally. At this time, the user needs to contact the original factory for the main board for replacement.